Triple Band, 30dBm

Power Repeater

850-1700-1900MHz

Cellular - AWS - PCS

Model No: PR-85-17-19-30W80

Features:

- Supports CDMA, GSM, WCDMA & LTE standards with 30dBm composite power per band

- Customized bandwidth (factory setting) per customer definition

- User friendly Repeater computer GUI

- Isolation detection function

- Linear power amplification to effectively suppress inter-modulation and spurious emission

- Smart Automatic Level Control (ALC) ensures output level stable and adjustable continuously

Introduction

The EMTS telecom Services Ltd. Tri Band Selective Power Repeaters (PR) provides an excellent solution to the problem of poor signal coverage for coverage extension and for in building & outdoor applications. The PR designed to support signal coverage for three bands 850, 1700 and 1900MHz. The repeater amplifies the signals from mobile phones and base stations and can be used in dead areas where service is poor. The PR is also improving the capacity of the cellular network and the data coverage at areas with limited signal strengths. The unit is connected to an outdoor donor antenna using a coaxial cable. The donor antenna transmits signals from mobile phones and receives signals from the BTS service antennas are placed in dead zones or at the tower for outdoor applications. Signals from mobile phones used in the dead zones are then received by the service antenna and transmitted to the PR donor antenna, which in turn transmits the signals to the base station.

Technical Specifications:

| - Electrical Specification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cellular 850 & PCS 1900 |  | Uplink |  | Downlink |
| Frequency Range | C850 Band | 824-845 MHz |  | 869-894 MHz |
|  | P1900 Band | 1850 -1910 MHz |  | 1930-1990 MHz |
| Bandwidth |  | Factory pre set Contiguous band 5-25 MHz |  |  |
| Maximum Gain |  | ≧80dB |  |  |
| Gain Flatness |  | ≦3dB(P-P) |  |  |
| Output Power |  | 25dBm ±2dB |  | 30dBm ±2dB |
| Out of Band Gain | ±1MHz | < 35dB |  |  |
|  | ±5MHz | < 25dB |  |  |
| Intermodulation Products | 9KHz~1GHz | ≦ -36dBm |  |  |
|  | 1GHz~12.75GHz | ≦ -30dBm |  |  |
| Spurious Emission | 9KHz~1GHz | ≦ -36dBm |  |  |
|  | 1GHz~12.75GHz | ≦ -30dBm |  |  |
| AWS 1700 |  | Uplink |  | Downlink |
| Frequency Range |  | 1710-1755 MHz |  | 2110-2155 MHz |
| Bandwidth |  | Factory pre set Contiguous band 5-25 MHz |  |  |
| Maximum Gain |  | ≧80dB |  |  |
| Gain Flatness |  | ≦2dB/3.84MHz |  |  |
| Output Power |  | 25dBm ±2dB | 30dBm ±2dB |  |
| Out of Band Gain | 2.7≦f_offset＜3.5MHz | <60dB |  |  |
|  | 3.5≦f_offset＜7.5MHz | <45dB |  |  |
|  | 7.5≦f_offset＜12.5MHz | <45dB |  |  |
|  | 12.5≦f_offset | <35dB |  |  |
| Spurious Emission | 9kHz~150KHz | ≦ -36dBm/1 kHz |  |  |
|  | 150kHz~30MHz | ≦-36dBm/10 kHz |  |  |
|  | 30MHz~1GHz | ≦-36dBm/100 kHz |  |  |
|  | Fhigh + 10 MHz ~12.75 GHz | ≦ -30dBm/1 MHz |  |  |

| ACRR | 20dBc/30KHz@±5MHz |
| --- | --- |
|  | 20dBc/30KHz@±10MHz |
| Frequency Stability | ≦±0.01ppm |
| Error Vector Magnitude | ≦ 12.5% |
| Peak Code Domain Error | ≦-35dB@Spreading Factor 256 |
| Output Intermodulation | the output intermodulation level shall not exceed the Spectrum Emission Mask and the spurious emission requirements in this specification |
| Cellular 850, PCS 1900 & AWS 1700 |  |
| Manual Gain Control | 31dB in step of 1dB |
| Automatic Gain Control | ≧25dB |
| Max. Input Power Without Damage | ≧0dBm |
| Noise Figure | ≦8dB |
| VSWR | ≦2.0 |
| Group Delay | ≦ 5μs |
| Power Consumption | ≦ 65W |
| Power Supply | AC 110/ 220 V, 50/ 60 Hz |
| 2.Mechanical specification |  |
| Dimensions | 453.6*357.8*159.7 mm |
| Weight | ≦ 22kg |
| I/O Connector/ Impedance | N-female / (Nominal)50Ω |
| 3.Environmental specification |  |
| IP Rating | IP55 |
| Operating Temperature | -10℃~50℃ |
| 4.Monitoring specification |  |
| Alarm | (Optional via GPRS modem) |
| Local Control | Local Control Via RS23 |

Ordering information:

Model Number: PR-85-17-19-30W80-X-Y-Z

X = 850 Downlink center frequency

Y= 1700 Downlink center frequency

Z= 1900 Downlink center frequency

Note: All specifications subject to change without notice.