Cellular 850 – AWS 1700 MHz Dual-Band Adjustable Band Selective 30 dBm Power Repeater

Model No: PR-85-17-30W80

Features

- Adjustable band or Channeled selective; for dual band, Cellular 850 MHz and AWS 1700 MHz networks with 30 dBm composite per band.

- Compatible with GSM, LTE and WCDMA standards.

- Low Noise figure& excellent linear RF performance

- Automatic Level Control (ALC) ensures output level stable and adjustable continuously.

- High selectivity for excellent out of band signal rejection by Digital technology

- Isolation detection & Oscillation suppression feature

- Low power consumption & sleeping mode (saving power)

- remote monitoring through wireless modem

Introduction

The EMTS Telecom Services Ltd. Adjustable Band Selective repeater model PR-85-17-30W800 is an integrated dual band RF Repeater equipped with a tunable bandwidth configuration (Cellular 850 & AWS 1700 MHz). This tunable band selective system is a compact, lightweight & easy to install. It enhances coverage in indoor area of up to 5,000 sq. meters or extend the range for outdoor deployments. The unit is designed to expand signal coverage to areas where signal is weak or unavailable, e.g.: hotels, exhibition centers, basements, shopping malls, offices, parking lots, etc. Its tunable bandwidth capability enable operator to set the bandwidth during the installation. Its easy installation and maintenance can help carrier get fast return on investment, the repeater is working as a relay between the BTS and mobiles. It picks up the strongest signal from BTS via the Donor Antenna, linearly amplifies the signal and then retransmits it via the Indoor Signal Distribution System to the weak/blind coverage area and the mobile signal is amplified and retransmitted to the BTS via the opposite direction. The band-selective function can choose to amplify the signal within the customized band. The system supports LTE, GSM and WCDMA modulations.

Technical Specifications:

| Electrical Specifications |  |  |  |
| --- | --- | --- | --- |
| Cellular 850 MHz Band |  | Uplink | Downlink |
| Frequency Range | Band | 824-849 MHz | 869-894 MHz |
|  | Bandwidth | adjustable band or channel selective 0.8-18 MHz or Fixed Bandwidth (factory set) types |  |
| Gain |  | ≧80dB | ≧80dB |
| Gain Flatness |  | +/- 1 dB for any 3.84MHz |  |
| Output Power |  | 24 dBm±1dB | 30 dBm±1dB |
| Out of Band Gain | ±400KHz | < 50dB |  |
|  | ±600KHz | < 40dB |  |
|  | ±1MHz | < 35dB |  |
|  | ±5MHz | < 25dB |  |
| Intermodulation Products | 9KHz~1GHz | ≦ -36dBm |  |
|  | 1GHz~12.75GHz | ≦ -30dBm |  |
| Spurious Emission | 9KHz~1GHz | ≦ -36dBm |  |
|  | 1GHz~12.75GHz | ≦ -30dBm |  |
| AWS 1700 MHz Band |  | Uplink | Downlink |
| Frequency Range | AWS Band Extended AWS Band | 1710 – 1755 MHz 1710 – 1770 MHz | 2110 – 2155 MHz 2110 – 2170 MHz |
|  | Bandwidth | Adjustable bandwidth or Fixed Bandwidth (factory set) types |  |
| Maximum Gain |  | ≧80dB | ≧80dB |
| Gain Flatness |  | +/- 1 dB for any 3.84MHz |  |
| Output Power |  | 24 dBm±1dB | 30 dBm±1dB |
| Out of Band Gain | 2.7≦f_offset＜3.5MHz | <60dB |  |
|  | 3.5≦f_offset＜7.5MHz | <45dB |  |
|  | 7.5≦f_offset＜12.5MHz | <45dB |  |
|  | 12.5≦f_offset | <35dB |  |
| Spurious Emission | 9kHz~150KHz | ≦ -36dBm/1 kHz |  |
|  | 150kHz~30MHz | ≦ -36dBm/10 kHz |  |
|  | 30MHz~1GHz | ≦ -36dBm/100 kHz |  |
|  | 1 GHz ~ Flow - 10 MHz | ≦ -30dBm/1 MHz |  |
|  | Flow - 10 MHz ~ Fhigh + 10 MHz | ≦ -15dBm/1 MHz |  |
|  | Fhigh 10 MHz-12.75 GHz | ≦ -30dBm/1 MHz |  |
| ACRR |  | 20dBc/30KHz@±5MHz |  |
|  |  | 20dBc/30KHz@±10MHz |  |
| Frequency Stability |  | ≦±0.01ppm |  |
| Error Vector Magnitude |  | ≦ 12.5% |  |
| Peak Code Domain Error |  | ≦-35dB@Spreading Factor 256 |  |

| 850/1700 |  |
| --- | --- |
| Manual Gain Control | 31dB in step of 1dB |
| Automatic Gain Control | ≧20dB |
| Max. Input Power Without Damage | ≧0dBm |
| Noise Figure | ≦ 6dB |
| VSWR | ≦2.0 |
| Group Delay | ≦ 8μs |
| Power Consumption | ≦ 150 W |
| Power Supply | AC110/ 220 V, 50/ 60 Hz |
| Mechanical specifications |  |
| Dimensions | 410*320*95 mm |
| Weight | ≤ 30 kg |
| I/O Connector/ Impedance | N-female (Optional 7/16) / )50Ω (Nominal ) |
| Environmental specifications |  |
| IP Rating | IP65 |
| Operating Temperature | -30 to 50℃ |
| Monitoring specifications |  |
| Monitoring | UL/DL Gain |
|  | UL/DL PA Switch |
| Control | UL/DL ATT |
| Alarm | UL/DL AGC Out of Range |

Ordering information:

- For adjustable bandwidth use Model No: PR-85-17-30W80-T

- For Fixed bandwidth use Model No: PR-85-17-30W80-X-Y

X=850 MHz BW, Y=1700 MHz BW

All rights reserved

Note: All specifications subject to change without notice.