High Power Tri-Band

900-1800-2100MHz

Fiber Optic DAS System

Model No: FOR-91821-40W55

Features:

- Advanced Tri Band 900, 1800 & 2100MHz

- Supports Multi-band, Multi-operator and Multi-technologies GSM/WCDMA/LTE

- Connected direct to BTS or RF antenna

- Comply with ETSI, 3GPP & LTE standards

- 40dBm Downlink power per band (Optional: RF output 15- 40dBm), designed for in building or outdoor deployments

- Easy field installation and maintenance, reduces rollout and operational costs

- Advance monitoring systems with the possibility to send alarms via fiber cable from master to slave or via GSM modem

- 4 Remote units per master unit, designed to connect to POI (Point of Interconnection,) to support multi operator deployments.

- Compact Size in IP65 enclosure (Aluminum-alloy)

Introduction

The EMTS Telecom Services Ltd. Multi-band, Multi -Operator Fiber Optic Repeater system is the perfect solution to extend and improve the in building coverage area of 900/1800/2100MHz cellular systems in the campus area, tunnels, hospitals, shoppin malls and outdoor sites. The Fiber Optic Repeating system consists of Master and Remote (slave units ) with up to four slaves at one master unit. The Master unit is designed to be connected to a POI (point of interconnection) to support multi operator deployments, while the Slave module provides coverage in the service area and connected to the master unit via dark fiber. Fiber Optic Repeaters amplify in directions, uplink and downlink a continuous bandwidth, factory tuned. Its rugged construction and easy field maintenance, reduce operational costs and ensures a high MTBF. The EMTS Fiber Optic Repeating system is a cost-effective and practical solution for extending signal coverage in subways, tunnels and Indoor or outdoor distribution systems.

Technical Specifications

|  |  | GSM/WCDMA/LTE |  |  | WCDMA/LTE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Downlink |  | Uplink | Downlink | Uplink |
| Frequency Range |  | 935-960MHz 1805-1880MHz |  | 890-915MHz 1710-1785MHz | 2110-2170MHz | 1920-1980MHz |
| Output Power |  | 40±2.0Bm |  | -10±2.0dBm | 40±2.0Bm | -10±2.0dBm |
| Max Gain |  | 55±3dB |  |  | 55±3dB |  |
| Gain Modulation Range |  | 30dB |  |  | 30dB |  |
| Gain Modulation Step |  | 1dB |  |  | 1dB |  |
| Gain Modulation Error |  | for gain 0-20dB,Error≤1dB; for gain 21-30dB Error≤1.5dB |  |  | for gain 0-20dB,Error≤1dB; for gain 21-30dB Error≤1.5dB |  |
| Noise Factor |  | / |  | ≤5dB | / | ≤5dB |
| Max NO Damage Input Power |  | 10dBm |  | -10dBm | 10dBm | -10dBm |
| Automatic Level Control (ALC) |  | Output power variation < 2dB or be off when adding 10dB at max output power. |  |  | Output power variation < 2dB or be off when adding 10dB at max output power. |  |
| Pass Band Ripple |  | ≤3dB |  |  | ≤2dB/3.84 MHz |  |
| VSWR |  | ≤1.5 |  |  | ≤1.5 |  |
| Time Delay |  | ≤5.0μs |  |  | ≤5.0μs |  |
| IMD | In-Band | ≤-40dBc |  |  | / |  |
|  | Out-Band | 9kHz~1GHz: ≤-36dBm/30kHz |  |  | / |  |
|  |  | 1GHz~12.75GHz: ≤-30dBm/30kHz |  |  | / |  |
| ACLR(PAR=8dB) |  | / |  |  | ≤-45dBc@5MHz ≤-50dBc@10MHz |  |
| Spurious Emissions |  | In-Band | ≤-36dBm/3kHz or ≤-60dBc/3kHz |  | Meet 3GPP |  |
|  |  | Out-Band | 9kHz～1GHz：≤-36dBm |  |  |  |
|  |  |  | 1GHz～12.75GHz：≤-30dBm |  |  |  |
| Frequency Stability |  | ≤0.05ppm |  |  | ≤0.01ppm |  |
| PCDE |  | / |  |  | ≤-35dB |  |
| Evm(RMS) |  | / |  |  | 12.5% |  |
| RF Connection Mode |  | N/F,50Ω |  |  |  |  |
| Type of Fiber Optical Cable |  | Single Mode |  |  |  |  |
| Fiber Optical Interface |  | FC/PC |  |  |  |  |
| Fiber Optical Loss |  | Max 8dBo |  |  |  |  |
| Remote Monitoring & Control |  | GSM MODEM |  |  |  |  |

| Power Supply Mode | Master Unit : DC-48VDC Remote Unit : AC 110-220V |
| --- | --- |
| Power Consumption | Master Unit : ≤40W Remote Unit : ≤350W |
| Optical Wavelength | 1310nm/1550nm |
| Number of Remote Units | 4 Remote Units per 1 Master Unit |
| Operation Temperature | -25℃ to +55℃ |
| Casing class | Master Unit Unit:IP40 Remote Unit Unit:IP65 |
| Machine Weight | Master Unit: ≤15kg Remote Unit: ≤35kg |
| Dimension | Master Unit:483mm×133mm×400mm （W*H*L） Remote Unit:490mm×410mm×187mm （W*H*L） |
| MTBF | ≥100000h |