GSM-R 900 MHz

40dBm Band Selective

High Power Repeater

Model No: HPR-GR-900-40W90

Features:

## Supports GSM-R with 40 dBm downlink composite power

## Light weight, compact Size and high Performance

## Comply with Railway Telecommunications (RT); ER-GSM frequencies; Part 1: ER-GSM additional radio aspects, ETSI TS 102 281 V3.0.0 (2016-02)

## Cost effective alternative to exclusive BTS solution

## User friendly GUI & plug and play installation , Isolation detection function

## Low power consumption, Linear power amplification to effectively suppress inter-modulation and spurious emission

## Smart Automatic Level Control - Ensures output level adjustable continuously

Description

The EMTS Telecom Services Ltd.  High Power Repeater (HPR) is designed to provide a cost-effective solution, rather than adding a new Base Transceiver Station (BTS), to improve signal coverage and communication quality in GSM-R system. Its easy installation and maintenance helps carriers to get fast return on investment (ROI). The repeater is working as a relay between the BTS and mobiles. It receives the low-power signal from BTS via the Donor Antenna, linearly amplifies the signal and then retransmits it via the Service Antenna to the weak/blind coverage area. The EMTS Telecom Services Ltd.  Power-Repeaters provides an excellent solution to the problem of poor signal coverage. The Repeaters amplify the signals from mobile phones and base stations and can be used in dead areas where service is poor. GSM-R systems are used extensively for railway communication both in Europe and Asia. Our GSM-R repeaters are deployed to provide coverage within trains, tunnels and other confined spaces. Our GSM-R produces are easily configured either locally or remotely via a remote or a local , the products supplied in a weatherproof IP65 cases. This combination of repeaters permits to reduce dramatically the number of BTS, and then, the total cost of deployment, for a better Quality of Service. The benefits are double: better QoS and better total cost

Technical Specifications:

| Electrical Specifications |  | Uplink | Downlink |
| --- | --- | --- | --- |
| Frequency Range |  | Type A: 873–880 MHz Type B: 885–889 MHz | Type A: 918–925 MHz Type B: 930–934 MHz |
| Bandwidth |  | 0.2-7 MHz |  |
| Gain |  | ≥ 90 dB | ≥ 95 dB |
| Manual Gain Control (MGC) |  | 31 dB in step of 1 dB |  |
| Auto Gain Control |  | ≥ 20 dB |  |
| In band ripple |  | +/- 1.5 dB max. |  |
| Max. Input Power Without Damage |  | 0 dBm |  |
| Composite Output Power |  | 27±1dBm | 40±1dBm |
| Out of Band Rejection | ± 400 KHz | ≥ 25 dBc |  |
|  | ± 600 KHz | ≥ 35 dBc |  |
|  | ± 1 MHz | ≥ 40 dBc |  |
|  | ± 5 MHz | ≥ 50 dBc |  |
| Intermodulation Product |  |  |  |
| Spurious Emission | 9 KHz~1 GHz | ≤ -36 dBm |  |
|  | 1 GHz~12.75 GHz | ≤ -30 dBm |  |
| Noise Figure |  | < 5 dB |  |
| VSWR |  | 1.5:1 |  |
| Group Delay |  | < 5 µS |  |
| Power Supply |  | -48 v DC |  |
| Environmental specification |  |  |  |
| Environmental Conditions |  | IP65 |  |
| Humidity |  | ≤95%(no dew) |  |
| Operating Temperature |  | -25 ~ 55 ℃ |  |
| Mechanical specification |  |  |  |
| RF Connector |  | N-Female |  |
| Dimensions (W x H x D) |  | 428 x 328 x 220 mm |  |
| Weight |  | ≤ 25 kg |  |
| Monitoring specification |  |  |  |
| Local Control |  | USB port for local / remote Real-time alarm for door status, temperature, power supply, LNA, VSWR, self-oscillation, etc; |  |
| NMS Monitoring Function (Optional) |  | Remote control such as turn on/off, increasing/decreasing output power etc Real-time status for output/input power, UL/DL gain, all status of repeater etc.; |  |

Ordering information:

Model No.: HPR-GR-900-40W90-A-B

For Fixed bandwidth use model HPR-900-40W90-A-B.   A=Downlink center Frequency, B=bandwidth in MHz