UHF/LMR/ PMR  400MHz

High Performances

Fiber Optic Repeating System

Model No: FOR-FBDA-400-AW60

Features:

- Fiber Optic RF Repeater is a reliable solution to extend and improve the coverage area of UHF/LMR/PMR Networks, consists of two main modules, Master and multiple Slave units

- Supports all combinations of UHF/LMR/PMR 400 MHz bands.

- 33, 37, 40, 43dBm composite power types, meet LMR/PMR system standards

- Easy field installation and maintenance reduces rollout and operational costs

- The signal transmission in fiber optic repeater would not be disturbed by outside influences

- Provide quick RF coverage service to your LMR/PMR Base-Station

- Compact Size and High Performance in waterproof enclosure suitable for outdoor and indoor installations

- Remote unit design for wall mounting & Base unit design for 19 inch rack

Introduction
The EMTS Telecom Services Ltd. Fiber Optic Repeating system is the best and more reliable solution to extend and improve the coverage area of LMR 400MHz (TETRA, APCO-25 or Analog systems) telecommunication systems. The Fiber Optic Repeating system consists of two modules, Master and Slave. The Master module can be used directly coupled from BTS or can be connected to a RF Repeater receiving signals from the BTS, while the Slave (Remote) module provides coverage in the service area. Fiber Optic Repeaters amplifies in directions, uplink and downlink a continuous bandwidth, factory tuned. Its rugged construction and easy field maintenance reduces operational costs and ensures a high MTBF. The EMTS Fiber optic repeating system is a cost-effective and practical solution for extending signal coverage in subways, tunnels and Indoor or outdoor distribution systems.

Technical Specifications

| Specifications |  |  | Downlink | Uplink |  |
| --- | --- | --- | --- | --- | --- |
| Frequency Range Any partial bandwidth at the LMR 400MHz band (please specify uplink and downlink bands) |  |  | 380-512 MHz | 380-512 MHz |  |
| output power types |  | 2W | 33dBm | -20~0dBm |  |
|  |  | 5W | 37dBm | -20~0dBm |  |
|  |  | 10W | 40dBm | -20~0dBm |  |
|  |  | 20W | 43dBm | -20~0dBm |  |
| Gain control range |  |  | ≥30dB |  |  |
| Gain |  |  | 60 to 65 db | 55 to 60 dB |  |
| Gain control linearity |  |  | ±1dB |  |  |
| ALC |  |  | P0≤2dB |  |  |
| In-band ripple |  |  | ≤3dB |  |  |
| Inter-modulation attenuation (In-band) |  |  | ≤-40 dBc/30KHz |  | ≤-50 dBc/30KHz |
| Inter-modulation attenuation (out-band) | 9kHz ～ 1GHz |  | ≤-36dBm/30KHz |  |  |
|  | 1GHz ～ 12.75GHz |  | ≤-30dBm/30KHz |  |  |
| Spurious |  |  | ≤-22dBm/100KHz |  |  |
| Spurious (out-band) | 9kHz ～ 150kHz |  | ≤-36dBm/1KHz |  |  |
|  | 150kHz ～ 30MHz |  | ≤-36dBm/10KHz |  |  |
|  | 30MHz ～ 1GHz |  | ≤-36dBm/100KHz |  |  |
|  | 1GHz ～ 12.75GHz |  | ≤-30dBm/1MHz |  |  |
|  | 825MHz ～ 835MHz |  | ≤-47dBm/100KHz |  |  |
|  | 870MHz ～ 880MHz |  | ≤-47dBm/100KHz |  |  |
|  | 1.71GHz～ 1.92GHz |  | ≤-47dBm/100KHz |  |  |
|  | 3.4GHz ～ 3.53GHz |  | ≤-47dBm/100KHz |  |  |
| Out-band rejection | Per frequency band |  | Offset of working frequency band≥2.5MHz：≤ -40 dBc or≤-13dBm/30KHz |  |  |
|  |  |  | Offset of working frequency band≥10MHz：≤-60dBc or≤-33dBm/30KHz |  |  |
| Noise figure |  |  | ≤5dB |  |  |
| Propagation delay |  |  | ≤5μs |  |  |
| VSWR |  |  | ≤1.4 |  |  |
| Frequency tolerance |  |  | ≤±0.05ppm |  |  |

| External connection |  |
| --- | --- |
| RF Connector | N-F / 50 ohms |
| Fiber Connectors | SAPC |
| Number of Remote units from the Optical Base Unit | 8 |
| Alarm Detection | HPA, LNA, TEMP, PSU |
| Local Alarm Option; NMS interface | PSU, HPA, LNA, control gain of Uplink and Downlink |
| Power Supply | 110VAC or 220VAC ±15％ 50-60Hz Optional -48 VDC |
| Enclosure | Remote unit design for wall mounting, Base unit design for 19-inch rack. Remote Units case IP65 size 650×400×295 mm |
| Fiber type included | recommended Single mode, WDM |
| Wavelengths | 1310 and 1550 uM |
| Environmental |  |
| Operating Temperature Range | -20 to +55 °C |
| Cooling | Convection |
| Environmental Sealing | IP65 |
| Operating Humidity | Up to 95% (non-condensing) |
| Complies with | EN 301 489-18, ETSI TS 101 789-1, EN 60 950 |
| Standards | Locked aluminum wall mount case 650×400×295 mm |

Ordering information:

Model No: FOR-FBDA-400-AW60-X-Y

A=Downlink Composite Power can be 33, 37, 40 or 43

X= Uplink band

Y= Downlink band

All specifications subject to change without notice.